Instructions for fitting a bill reader to an N-Range machine.

- Ensure you have the necessary parts – see the “bill reader information” file in the technical support section of the website for a summary of parts.

- Power down machine;

- Ensure the EPROM on the control board (board with red LED) is a “-5” EPROM or later (i.e. -6 or -7). If not you need to replace it.

- Fix bezel (red note entry plate) to the supplied metal plate with bill-reader cut-out.

- Remove the blank plate from the front of the machine and replace with the new note entry plate.

- Plug note reader loom into note reader and fix the reader to the note entry plate.

- Ensure the machine has a note reader board fitted. (If this board is in the machine, there should be 4 boards instead of the normal 3 in the board housing – the board has a 15-way white output socket at the top). If this board is not there you will need to install one to the left of the existing boards.

- Fix the opposite end of the note reader loom to the note reader board on the 15-way output socket.

- Power on the machine.

- Go to programme #455 on the basic menu. If this programme is not there, access it through function 000 by holding down button C on the handset and then entering in “455” using buttons A, B & C. Press reject to enter the programme.

- “No bill reader” should appear. Change this to “Smiley 4T” by pressing button A. Then press the reject button repeatedly until “455 bill reader” appears again (do not change any of the other settings that will appear).

- Go to programme #221 “multi-vend” and change this to “yes” by pressing button A.

- Go to programme #224 “Over-vend” and ensure it is set at “no”.

- Go to programme #228 “age token” and make sure it is set at “no” or “free”.

- Go to programme #206 (by holding down button C until “function 000” appears and then entering in #206 using buttons A, B & C and pressing reject to confirm). “Press selection” will appear. Press the first selection button and “token – yes” will appear. Change this to “no” using button A on the handset and confirm with button D on the handset. Repeat this process for each selection button on the machine. (This switches off the token on each individual selection). Press reject to exit the programme.

- Test to ensure notes are registering the correct values.

During use:

If you wish to check what money should be in the note reader at any point, go to programme #149 “money in notes” by holding down button C . Press D to enter the programme. Use programme # 171 “count reset” to clear the data if you wish.