Instructions for replacing an EPROM in an N-range machine:

- Turn off the machine;

- Remove the old EPROM from the control board (3rd board from the right with a red LED) and insert the new EPROM. Ensure that the notch at one end of the EPROM is matched up with the notch on the EPROM socket on the board;

- Turn on the machine;

- Hold down button C on the handset until “function 000” appears on the display;

- Using buttons A, B and C input programme # 474 and press reject;

- “Access Level” will appear on the display. Use button A to increase the access level to 2. Press reject.

- “Password” will now appear on the display. Press reject;

- Hold down button C on the handset until “function 000” appears on the display;

- Input programme #99 and press reject. The machine will now perform a number of internal processes during which various symbols will appear on the display;

- Wait until “autocfg-col yes?” appears on the display. Press reject;

- The machine will now check the positioning of all motors. This may take a number of minutes. Any motors on which there is no stock will perform a rotation. When the process is complete, the machine will reboot itself and “out of service” will appear on the display;

- Now set up the Basic Menu. This is where you decide which programmes from the master memory of the machine are to appear in the normal listing of programmes that you would normally access by pressing button A on the handset when the machine is in normal operation;

- Hold down button D on the handset for several seconds until there is a rapid beeping and then let go. “440 Basic Menu” will appear on the display.

- Press D. Each programme in the master memory of the machine will now appear on the display beginning with programme #1 “empty hopper”. Use buttons A and B on the handset to scroll through the programmes. Beside each programme “yes” or “no” will be displayed.  Button D will toggle between yes and no. Setting a programme as “yes” means it will appear in the programmes when you access them normally using button A.

- When you have finalised which programmes are to appear in the basic menu press C on the handset twice to return to normal mode.

- You will now need to programme up the machine in the normal way including:

  - Setting the prices (programmes 201, 202 or 204);

  - Setting the overvend (programme 224);

  - Linking the columns (programme 300);

  - Turning on or off tokens (programmes 206 and 228);

  - Bill readers only - Turning on multivend (221) and bill reader (455)

NB – if any motors are faulty or not connected properly they will not be detected by the machine during the reset process and the column linking will be misaligned. If this occurs you must replace the faulty motor and repeat the above process.