S-Range with digital display

Price program.

- Press program button once (see picture below).

- 202 will appear on screen.

- Insert coins to exact amount of price.

- Press each selection button you want at that price.

- Reject coins

- To set different price insert coins to exact amount of price.

- Press each selection button you want at that price.

- Press program button until you get 000.

Programming button

Overvend.

- Press program button twice.

- 224 will appear on screen.

- Insert coins to amount you want to overvend

- Press program button until you get 000.

S-Range without digital display

Price program.

- Press program button once – the green pilot light on the front of the machine will flash slowly.

- Enter the price of the first selection button in coins and then press reject;

- Press the 2nd selection button and enter the price in coins again and press reject;

- Repeat for all selections;

- When you are finished, press the programming button once only to return the machine to normal working mode;

Overvend.

- Press program button twice – both the red and green pilot lights on the front of the machine should now be flashing very quickly;

- Insert coins to amount you want to overvend

- Press program button once to return the machine to normal working mode.

- When and only when the green pilot light is on continuously and the red light is off, you can reject any coins that may be in the escrow.