# Token Instructions

Instructions for switching on age control tokens on Azkoyen machines:

## N - Range

The procedure to be used will depend on which Eprom is in the machine. You can identify which EPROM is in the machine by watching the display as the machine is powered up. When you turn on the machine, the numbers “43521560” and then “-5” or “-6” or “-7” will appear briefly. This indicates the type of EPROM in the machine.

Older Eproms (-5 or earlier)

To turn on tokens:

Press and hold button ‘C’ for a few seconds until the beep increases in speed. <<FUNCTION 000>> appears, enter ‘228’ (Press ‘A’ to increase units, ‘B’ to increase tens and ‘C’ to increase hundreds) press reject when entered. ‘Adult Token?’ will appear with ‘Yes’ or ‘No’ beside it, press ‘A’ to select ‘YES’ or ‘NO’ and Reject button to set this. The machine will restart automatically.

To turn off tokens:

Press and hold button ‘C’ for a few seconds until the beep increases in speed. <<FUNCTION 000>> appears, enter ‘228’ (Press ‘A’ to increase units, ‘B’ to increase tens and ‘C’ to increase hundreds) press reject when entered. ‘Adult Token?’ will appear with ‘Yes’ beside it, press ‘A’ to change this to ‘NO’ and Reject button to set this. The machine will restart automatically.

Newer Eproms (-6 or later)

Firstly carry out the procedure described under the “-5” instructions above. In addition to switching on adult token using programme #228 as described above, you also need to turn on the token for each individual selection button as described below. NB – this is in addition to the procedure described above under “older Eproms”.

- When the above procedure (as described under “-5” above) is complete, go to function #206 by pressing and holding button ‘C’ for a few seconds until the beep increases in speed. <<FUNCTION 000>> appears, enter ‘206’. When you have entered the numbers “206”, press Reject to enter the programme;

- “press selection” will appear on the screen. Press the first selection button and either “yes” or “no” will appear on the screen depending on whether tokens are turned on (yes) or off (no);

- Press button A so that “yes” appears and then press D to confirm;

- Now press the next selection button and repeat the above;

- When all selections have been switched to “yes” press the reject button to exit that programme;

- Note that the word “token” should now appear instead of the price under each selection button.

- If you only wish to turn on tokens for the purpose of testing the token in the coin validator then only one selection needs to be turned on for tokens.

- To turn off tokens – Firstly carry out the procedure for turning off tokens as described under “older EPROMS” above; then go back into function #206 and ensure that tokens are turned off for each selection button – ensuring that if you change any selections from “yes” to “no” that you press D on the handset to confirm.

## Panoramic Range

Press button ‘A’, ‘Pr 0’ will appear on the display. Press reject button repeatedly until ‘Pr96’ appears on the display. Enter the password by pressing buttons ‘A’, ‘A’, ‘B’, ‘B’, ‘C’, ‘C’, ‘D, ‘D’, in that order, to enable the extra programs. Press reject button, ‘EEEE’ should flash on the display to show the password has been accepted. Press the reject button repeatedly until ‘Pr62’ appears. Press the rightmost selection button repeatedly until ‘No 13’ flashes on the display, a number, usually ‘0.00’will be displayed. Change this number to ‘99.99’ using the A, B and C buttons; C for the number before the decimal point, B for the first decimal place and A for the second decimal place. When ’99.99’ has been entered, press ‘D’ 3 times, ‘EEEE’ will flash on the display to show that the value has been saved. Press reject button repeatedly until ‘Pr 0’ is displayed and press ‘A’ to return to sales mode. Machine will then restart.

To turn off tokens:

Repeat the above procedure exactly except that you need to change the “99.99” as per the above to “00.00”.

## D - Range

Put switch at back of display to the down position. Press reject button repeatedly until ‘Inhibition’ appears, press ‘D’. Press ‘B’ repeatedly until ‘Token Adults’ appears with ‘on’ or ‘off’ beside it. Press ‘A’ to switch between ‘on’ and ‘off’, select ‘on’. Press button ‘D’ 4 times to save the settings. ‘Order Accepted’ should appear on the display for a second. Put switch at back of display to the up position.

To turn off tokens:

Repeat the above procedure exactly but when “Token Adults” appears with “on” or “off” beside it ensure to select “off”.  Press button ‘D’ 4 times to save the settings. ‘Order Accepted’ should appear on the display for a second. Put switch at back of display to the up position.

## S- Range (without digital display)

Press the button on the main board 3 times (wait a moment after each press), ‘On’ or ‘Sold out’ light, (not both), will flash quickly. The ‘On’ light flashing indicates the token is off, ‘Sold out’ indicates it is on. Press the leftmost selection button to select on, ‘Sold out’ light will flash quickly. Press the button on the board to return to sales mode, ‘On’ light will appear to indicate this.

To turn off tokens:

Repeat the above but use the leftmost selection button to make sure the “on” light is flashing.

## S- Range (with digital display)

Older Eprom (-0)

- Press the button on the main board (programming button) 3 times until “228” appears;

- Then press 1st selection button to turn tokens on (note if they are already on this will turn them off);

- The press the programming button until “000” appears.

- Repeat the above if you wish to turn off tokens.

Newer Eprom (-3)

- Press the button on the main board (programming button) twice until “206” appears;

- Then press each selection button twice to turn the token on (note if they are already on this will turn them off);

- Then press the programming button twice until “228” appears;

- Then press 1st selection button to turn tokens on (note if they are already on this will turn them off);

- The press the programming button until “000” appears.

- Repeat the above if you wish to turn off tokens.